A Collaborative VR Visualization Environment for                                       Offshore Engineering Projects

Ismael H. F. dos Santos

CENPES – Petrobras Research Center

Rio de Janeiro, Brazil

ismaelh@petrobras.com.br

Luciano Soares, Felipe Carvalho, Alberto Raposo

Tecgraf – Computer Graphics Technology Group

Pontifical Catholic University of Rio de Janeiro, Brazil

{lpsoares,kamel,abraposo}@tecgraf.puc-rio.br

Abstract

The current way of designing industrial plants relies in the communication among the experts on the area and tools that allow the simulation of the site.  Virtual Reality tools are used to visualize and interact with complex 3D environments in real time, and several engineering simulations are used to foresee the results of complex industrial operations. The research project presented here presents a collaborative environment engineering (CEE) that integrates virtual reality techniques into a collaborative environment where the execution of different sequences of engineering simulations is modeled as scientific workflows. The focus of this research is on the oil & gas industry, focused in the Offshore Engineering domain where the project of a new production unit is a lengthy and expensive process and conducted by diverse specialists geographically distributed.  Among the integrated engineering simulations it is possible to highlight the ones from structural calculus, hydrodynamics, naval engineering with mooring systems, meteo-oceanography, etc. Thus, CEE main objective is to improve users interpretation capacity and skills while providing visualization tools for a better comprehension of the results of executed simulations.

CR Categories: I.3.7 [Computer Graphics]: Three-Dimensional Graphics; Realism– Virtual reality; Graphics systems; Distributed network graphics;

Keywords: three-dimensional graphics; virtual reality; collaborative virtual environments; offshore engineering.

1. Introduction

In recent years, the oil & gas industry has seen increasing costs of finding and lifting hydrocarbon, especially in more remote locations, ultra-deep water reservoirs (400 m or deeper) or in hostile environments. The development of deep-water oil & gas reserves constantly faces the challenge of reducing costs of all components and activities in the selected exploitation scheme. Therefore, High Performance Computing, Visualization and Collaboration technologies are being heavily used to improve productivity leading to better cost-performance ratio.

Earth Sciences and Engineering are challenged to manage and interpret increasing amounts of data coming from the captured information of the environment or generated by computer simulations. The typical work of scientists and engineers consists in detecting features, measuring them, and finally generating a model that tries to explain those observed features. This visual approach to science and engineering is powerful, as the human brain excels at visually identifying patterns. As Edward Tufte [Tufte 1983] wrote more than two decades ago:  “At their best, graphics are instruments for reasoning about quantitative information. Often the most effective way to describe, explore and summarize a set of numbers – even a very large set – is to look at pictures of those numbers”.

Visualization and Remote Collaboration technologies help us to bridge the cost-productivity problem. High-end visualization systems are commonplace in the oil & gas industry. In former times, the industries have shown sensitive gains in efficiency and effectiveness when carrying out enterprise projects using Virtual Reality technologies. In the nineties oil companies were among the first to make industrial use of the so-called virtual reality centers, equipped with immersive projection systems with large display walls (e.g., cave, cave-like, curved-panel, and powerwall), videoconference tools, among other solutions Three-dimensional geometric modeling, scientific visualization and immersive virtual environments, commonly used in these facilities, pushed the limits of teamwork activities in Geosciences and Engineering.

VR visualization technologies enhance the content knowledge within any engineering design activity. When used in conjunction with collaboration, VR visualization provides valuable insights for better Decision Support with risk mitigation. [Dodd 2004] has mentioned that the next big management push is the empowerment of interdisciplinary teams with collaboration tools that include remote and immersive visualization on the desktop. Based on this, we emphasize that the combination of collaborative tools and VR visualization constitutes a powerful component for any software solution for large scale Engineering Projects.

The present work is motivated by the necessity of finding effective solutions for collaboration of team workers during the execution of large and complex Offshore Engineering projects. Those projects usually require the execution of a large number of engineering simulations, encapsulated as engineering services, combined in different orders and rearranged in different subsets according to project requirements comprising different scientific workflows [Deelman et al. 2009] especially tailored for the Offshore Engineering domain.

By means of a Scientific Workflow Management System users are able to orchestrate the execution of engineering simulations as workflow tasks that can be arranged in many different ways. Within a workflow, as its last step, the most interesting cases can also be selected for visualization in an immersive collaborative session. The distributed execution of simulations is supported by a simplified Grid computing infrastructure seamlessly integrated into the system. The collaboration support is provided by a videoconference system, a mechanism for creating private and shared 3D annotations connected to engineering artifacts, and additional visualization feedthrough mechanisms that improves the system awareness. Those features compose a Collaborative Problem Solving Environment that enables engineers to setup computations in an integrated environment [Houstis et al. 1997]. CEE is therefore composed of the following different tools for distributed group work:

- Virtual Reality Visualization tool for collaborative visualization of engineering simulations in an immersive or desktop environment;

- Scientific Workflow Management System based on BPEL (Business Process Execution Language) [OASIS] workflow implementation used as a process-oriented tool to control the execution of simulations;

- Videoconference System developed for supporting human communication, providing integrated audio and video channels, subjected to defined control policies;

In the following sections we present some aspects of the developed system. In section 2 we present the main characteristics of Offshore Engineering projects discussing some problems addressed. Related works that inspired the development of the system are presented in section 3. In section 4, we describe the main architecture of the CEE. Some application scenarios are presented in section 5 and conclusions in section 6 finish the paper.

2. Offshore Engineering Projects

In the Offshore Engineering domain the project of deep-water production systems, including floating production units (platforms or ships) and all the equipments playing a part in the production process, are currently designed by means of complex computational modeling systems. The project of a new production unit is a lengthy and expensive process, that can last many years and consume hundreds of million of dollars, depending on the complexity of the unit and how mature is the technology developed to make the project technically and economically feasible.

Usually Offshore Engineering projects involve not only geographically distributed teams but also teams of specialists in different areas using different software tools, both commercial and internally developed. The interoperability of those tools is still an issue in the industry and is a mandatory requisite for any viable collaborative solution.

Figure 1. Deep-water production system layout.

Due to their huge complexity, Offshore Engineering projects are divided into smaller interrelated subprojects where each one deals with an abstract representation of the others. Because decisions are interdependent the necessity for collaboration is a key issue in this area. Each team activity or new decision can affect other activities. For example, during the design of an oil platform, changing the position of large and heavy equipment in the process plant can compromise the stability of the production unit. In some cases there is also an intrinsic coupling among the solutions of the different subprojects, which requires a lot of interactions and discussions among the teams involved. This is the case of the mooring system and of the production risers’ subsystems (Figure 1). On one hand if the mooring system allows great fluctuations of the production unit, it can simply damage the production risers; on the other hand the presence of the risers itself helps to weaken the movements of the production unit that contributes positively to the equilibrium of the system. In order to achieve collaboration and interoperability between those subprojects a software-based interface is required.

A different problem, related to the necessity of multiple data representation, is that although the specialists deal with the same information (platforms, risers, mooring systems, etc.) they usually have different data representations for those objects according to the needs of each activity, requiring some support for multi-resolution representation of the data. For example, in structural and naval engineering the models usually have dense polygonal meshes, with a few objects representing the outline of the artifacts, suitable for numerical analysis of static and dynamic stability studies. Those different data representation imposes two alternatives: either implements support for multi-resolution representation or improves rendering performance in order to deal with real data. The second approach relief the solution from conversions between models.

Another challenge present in Offshore Engineering projects is related to the visualization of large engineering simulations. During the conceptual design phase of an industrial plant, several simulations should be applied to assess the robustness and feasibility of the project. Some of these simulations may require huge computational efforts to be processed, like computer clusters. Visualization should be as precise as possible in order to provide the user a full understanding of the results of the simulation.

In CAD/CAE the models usually have objects with coarse grid meshes suitable for good visual representation, but the problem is that all the objects that comprise the artifact are represented yielding huge models (millions of 3D objects). Another aspect is that the CAD/CAE models are the best representatives for real artifacts, so there is a trend of using those models directly in simulations and visualizations. Even today, realistic real time visualization of those models is a complex problem in computer graphics indeed [Removed due to blind review].

This research project focuses on the development and integration of scientific and visualization tools and technologies coupled with collaborative environments that support the modeling and simulation of complex scientific and engineering projects. CEE is designed to solve simple or complex problems providing support for  both rapid prototyping and detailed analysis,

In conclusion, according to the above challenges presented for Offshore Engineering projects, CEE was conceived as a useful solution for better controlling and executing specialized engineering projects through the use of its collaboration and visualization capabilities.

There are other important problems that can be tackled with CEE such as: training and security simulations; design, planning and optimization of marine installations and sub-sea layout arrangement of production equipments; remote operation and interventions on submarine equipments; preparing maintenance and inspections plans in production units; planning oil pipeline installation and monitoring and emergency scenario applications. In this paper, in section 5, we present the use of CEE for controlling and executing risers structural analysis project.

3. Related Work

In this section we present a few major works that motivated this research towards the construction of collaborative immersive visualization environment for visualizing engineering simulations.

[Parker et al. 1998] describe SCIRun, a Problem Solving Environment (PSE) that allows users to interactively compose, execute, and control a large-scale computer simulation by visually “steering" a dataflow network model. SCIRun supports parallel computing and output visualization, but originally has no mechanisms for experiment managing and archiving, optimization, real-time collaboration, or modifying the simulation models themselves.

Paraview is a kind of PSE for visualization that allows the interactive creation and manipulation of complex visualizations [Paraview 2011]. Paraview is based on the notion of dataflow, and provides visual interfaces to produce visualizations by assembling pipelines out of modules that are connected in a network. However, both SCIRun and Paraview have important limitations which hamper their ability to support the data exploration process. First, there is no separation between the definition of a dataflow and its instances. In order to execute a given dataflow with different parameters, users need to manually set these parameters through a GUI — clearly this process does not scale to more than a few visualizations. Second, modifications to parameters or to the definition of a dataflow are destructive — no change history is maintained. This places the burden on the scientist to first construct the visualization and then to remember the values and the exact dataflow configuration that led to a particular scenario. Despite their limitations, SCIRun and Paraview show the importance of combining visualization with PSE.

[Paventhan et al., 2006] proposed the creation of WindTunnel, a Scientific Workflow for wind tunnel applications. They observed that scientific and engineering experiments often produce large volumes of data that should ideally be processed and visualized in near real-time. The difficulty to achieve this goal is that the overall turnaround time from data acquisition, movement to a data processor and visualization of the results is frequently inhibited by factors such as manual data movement, system interoperability issues, manual resource discovery for job scheduling and disparate physical locality between the experiment and the user workstation. They argued that customized application specific workflows can reduce the time taken to accomplish a job by automating data flow driven activities, supplementing or replacing manual user-driven tasks. WindTunnel provides a series of workflow activities allowing the users to compose sequential workflows and seamlessly access Grid services. WindTunnel approach also inspired the development of the proposed system by combining the Scientific Workflow Management System (ScWfMS) with the execution of engineering applications in a Grid infrastructure.

Vistrails [Callahan et al. 2006] is a visualization management system that provides a Scientific Workflow infrastructure which can be combined with existing visualization systems and libraries. A key feature that sets Vistrails apart from previous Visualization Systems as well as Scientific Workflow Systems is the support for data exploration. It separates the notion of dataflow specification from its instances. A dataflow instance consists of a sequence of operations used to generate a specific visualization. Vistrails approach inspired the strategy of the software we have developed, but some of the differences from our solution are the use of a BPEL (Business Process Execution Language) ScWfMS [EclipseBPEL] and the focus on immersive and realistic visualization.

In the Geology field, [Kreylos et al., 2006] presented an approach for turning immersive visualization software into a scientific tool. They created immersive visualization software, with measurement and analysis tools that allow scientists to use their real-word skills and methods inside a VE. They emphasized that VR visualization alone is not sufficient to enable an effective work environment. This observation has also motivated the creation of some additional tools for the VR visualization subsystem proposed.

In the Upstream segment of the oil & gas industry, the determination of optimal well locations is a challenging problem for Reservoir engineers since it depends on geological and fluid properties as well as on economic parameters [Klie et al. 2004]. [Gruchalla et al. 2004], investigated the benefits of immersive VR for well-path editing. They reported speed and accuracy improvements of immersive systems over desktop system, based in a study with 16 participants who planned the paths of four oil wells. Each participant planned two well paths on a desktop workstation with a stereoscopic display and two well paths in a CAVE-like Immersive Virtual Environment [Cruz-Neira et al. 2002]. Fifteen of the participants completed well path editing tasks faster in the CAVE environment than in the desktop environment. The better speed was complimented by a statistically significant increase in correct solutions. The results suggest that an immersive environment allows for faster and more accurate problem solving in a complex interactive three dimensional domain.

The Fraunhofer Gesellschaft VRGeo Consortium [VRGeo website] is an oil & gas international consortium for developing visualization technology for Geosciences and Engineering applications in Virtual Environments. VRGeo has been presenting many significant contributions for the use of VR technology, especially in the area of Collaborative Work in Virtual Environments. [Simon et al, 2005] presented a qualitative and quantitative study comparing usability and interaction performance for multi-viewpoint images, where a large screen projection-based stereoscopic display system is shared by a small group of people, each of them with its own viewpoint.

Another work was the VRGeo Demonstrator Project for Co-located Collaboration interactive analysis of complex geological surfaces and volumes in an immersive VR system [Simon, 2005]. They showed a new interaction paradigm allowing multiple users to share a virtual space in a conventional single-view stereoscopic projection-based display system, with each of the users handling the same interface and having a full first-person experience in the environment.

The effective integration of “smart” graphical user interfaces, with some advisory support, Scientific Visualization, Virtual Reality techniques, Engineering Analysis and Modeling Tools aid in the automation of modeling analysis and data management for large and complex engineering projects. To enhance engineers ability to share information and resources with colleagues at remote locations, collaborative and real-time technologies integrated into CEE provided a unified approach to the scientific and engineering discovery and analysis process.

A combination of a collaborative problem solving environment and VR visualization, which is not addressed by the aforementioned visualization systems, constitutes a strategic enabler for successful data exploration and knowledge dissemination among workers in engineering enterprises. VR visualization technologies enhance the content knowledge within the engineering disciplines. In conjunction with collaboration, both provide valuable insights for better Decision Support with risk mitigation.

4. CEE Architecture

In order to achieve its goals CEE architecture is a composition of different Computer Supported Collaborative Work technologies to create a useful collaborative engineering environment. The system is composed of a Collaborative Visualization Environment based on a Virtual Reality Visualization (VRV) tool and a Videoconference System (VCS); a Scientific Workflow Environment with a Grid Computing Infrastructure (GCI) support for executing large engineering simulations; and a Project Management Environment (PME) responsible for controlling the overall execution of the project and keeping track of all the information and different artifacts generated during project’s entire lifecycle.

CEE allows users to collaboratively solve their problems through the use of predefined scientific workflows or assembling new ones as necessary. Each workflow comprises a sequence of simulations, usually ending with a collaborative visualization supported by a VRV tool.

and different artifacts generated during project’s entire life cycle.

Figure 2: Conceptual Model.

To achieve this, CEE was devised as an extensible platform, flexible and independent of system, allowing a transparent flow of information among different levels, systems and models. The challenges for building an effective and useful solution can be scrutinized according to the following aspects:

- Collaborative Work – effective human-to-human interaction and communication for solving conflicts and enhancing group productivity should be provided;

- Virtual Reality Visualization – high performance and scalability are important aspects of virtual environment architectures intended to support execution of large shared virtual worlds over long periods of time;

- Scientific Workflow Environment – challenges related to the control of the execution of engineering simulations such as interoperability and distributed execution as well as data provenance should also be provided;

- Project Management Environment – the ability to track all of the documents and artifacts generated during project's life-cycle. Multiple and different visions of the on-going project must be provided while users have different background.

The conceptual model (Figure 2) handles the above-mentioned challenges creating specific services for them composing the Service Oriented Architecture (SOA) solution.

The Collaborative Visualization Environment is responsible for managing the user interaction. The Videoconference Service and the VR Visualization Service work closely coupled with the Collaboration Manager Service to enable the creation of collaborative visualization sessions.

The Scientific Workflow Environment was created to help the users build engineering workflows and seamlessly execute them in a Grid Computing Infrastructure (GCI). More generally, to support distributed execution, we use the interoperability characteristics of the ScWfMS and the distributed execution support provided both by the GCI and also the support provided by the SOA backbone infrastructure furnished by the Enterprise Service Bus (ESB). For interoperability among applications it was developed a common format for data exchange among engineering applications.

The Engineering Simulations Service provides a Webservices interface [Lee et al 2005] for remotely execute an engineering simulation program. Some of those simulators are, Anflex [Mourelle et al 1995] a Finite Element riser analysis software, and Prosim a coupled analysis software for the design of floating production systems.

## 4.1 Scientific Workflow

[Ellis et al. 1999] presents Workflow Management Systems (WfMS) as a tool to assist in the specification, modeling, and enactment of structured work processes within organizations. These systems are a special type of collaboration technology which can be described as “organizationally aware groupware”.

Although the above definitions make reference to “Business Process”, WfMS is not only employed by business applications. In recent years, several industries have improved their operations through the use of Workflow Management Systems – improvement of data management and better coordination of activities through specific Business and Scientific and Engineering Process. However, there are remarkable differences between Business and Scientific workflows. [Weske et al. 1998] identified that in a scientific environment scientists will typically specify their workflows themselves, while in a business environment; a system administrator is commonly responsible for this task. Another characteristic of Scientific workflows pointed in their work is the need to trace workflow executions. An engineer may need to reuse a workflow in order to reproduce results. The operations a user performs on a given data must be recorded in order to provide engineers with the benefits of successful and unsuccessful workflows.

Scientific workflows often begin as research workflows and end up as production workflows. Early in the lifecycle, they require considerable human intervention and collaboration; later they begin to be executed increasingly automatically. Thus in the production mode, there is typically less room for collaboration at the scientific level and the computations are more long-lived. During the research phase, scientific workflows need to be enacted and animated (fake enactment) far more intensively than business workflows. In this phase, which is more extensive than the corresponding phase for business workflows, the emphasis is on execution with a view to design, and thus naturally includes iterative execution. The corresponding activity can be viewed as a “Business Process Engineering” (BPE). For this reason, the approaches for constructing, managing, and coordinating process models are useful also in scientific settings. In this way, Scientific Workflows are to Problem Solving Environments what Business Workflows are to Enterprise Integration.

## 4.2   Support for Collaborative Work

Collaborative systems should not only allow multiple users to interact with shared objects but also to communicate and to coordinate their actions. Collaboration may be seen as the combination of communication, coordination and cooperation [Ellis et al. 1991]. Communication is related to the exchange of messages and information among people. Coordination is related to the management of people, their activities interdependencies and the used resources. Cooperation is the production of common artifacts taking place on a shared space through the operations available to the group.

In CEE the support for collaborative work is provided by the use of a Scientific Workflow Management System (ScWfMS), Videoconference System and a Collaboration-Bus, a collaborative infrastructure support for integrating the execution of engineering applications with our Virtual Reality Visualization component allowing the users to collaboratively visualize their results and optionally create virtual Annotations, to share knowledge about the engineering artifacts being visualized.

The ability to collaboratively create persistent annotation on the model improves its usefulness. An annotation, in our context, is any textual information that users want to add to their projects to enrich the content or just for documentation; it can have a private or public (shared) scope. Annotations can be associated to any engineering artifact manipulated during a collaborative visualization session that usually happens at the end of execution of any sequence of engineering simulations, as the last step of typical engineering workflow (Figures 6 – 9).

An Annotation could, for example, represent a kind of instructional information denoting a sequence of operations that should be undertaken during an equipment maintenance intervention in a production unit. It can also be any textual information used to highlight interesting or anomalous events observed on the simulation results. Examples of such events could be unexpected values for an engineering quantity, violations of integrity, etc. Annotations can also have a more dynamic behavior, while they can represent distance measures between distinct objects that should be monitored during a simulation, e.g. the distance between two different elements in two different ascending risers to a floating production system. Some of the discussed examples can be seen in section 5.

Cooperation also occurs during the assembling of useful engineering workflows that will be used to orchestrate the execution of engineering applications, and also during visualization of results when the users can collaborate to better understand the model. Users can also share persistent annotations about interesting facts, as previously discussed.

There are different types of awareness support provided in CEE. In our scenario, the most important ones are:

- event monitoring – observes what is going on in the VRV, in all separate parts and provide active notification to the right person, at the right time and the right sub-system;

- workspace awareness – provides control of collaborative interaction and changing of the user location;

- mutual awareness – allows users see each other’s identity and observe each other’s actions;

- group awareness – facilitates the perception of groups of interest connecting people who need to collaborate more intensely. Group awareness enables the user to build his own work context and to coordinate his activities with those of others’. Informal communication provided by a Videoconference system enhances team awareness, User awareness is a very important subject for CEE once it is a desirable feature to provide mutual awareness during the collaborative visualization session allowing users see each other’s identity and observe each other’s actions specially when creating Annotations in the model.

The Collaboration Manager is responsible for managing the users’ participation in a collaborative session and also integrates the resources of Virtual Reality Visualization and Videoconference system. There are three kinds of sessions available: Informal, Classroom and Lecture.

Users can request the coordination role to the current coordinator who can accept or reject the request, generating a visual feedback to the requestor. Upon the occurrence of a “change coordinator” event in a CEE collaborative session, all users are notified by the CEE Awareness mechanism.

At any time the user can disconnect from the session, for doing some private work, and reconnect to session in later time, when its state is synchronized with the sate of the session, that is controlled by the Collaboration Manager Service.

The Collaboration Bus (CBUS) is a key component of the overall architecture and provides synchronous and asynchronous communication for the CEE components. The CBUS is an infrastructure for communication based on the JMS Service Provider, the Message Oriented Middleware (MOM) used for giving the public/subscribe and point-to-point paradigms, and the Enterprise Service Bus (ESB).

The CEE Awareness Service is an awareness service providing appropriate actuators for events received from the CBUS. It is responsible for signaling distributed events to the users participating in a collaborative session. In one side all components trigger events to this distributed bus, and in the other side awareness components listen to the bus for gathering information about what is happening in the system. For example, when users leave a collaborative session or when there is a change in its state from offline to online and vice-versa, “update user” events are triggered to the CBUS and the CEE Awareness mechanism send messages to Virtual Reality Visualization Service and Videoconference Service notifying the event. By their turn, those services signal those events in their user interfaces making the user conscious of what have happened. Examples of such kind o signaling used in CEE are: different icons in the GUI for showing the state of the users; windows messages warning the entrance of a new user in the session or the leaving of the user from the session, etc.

## 4.3   VR Visualization Tool

During the project a tool was developed to facilitate the use of massive CAD models in VR applications. It is a system composed of a 3D environment for real-time visualization and plug-ins to import models from other applications, allowing users to view and interact with different types of 3D data, such as refineries, oil platforms, risers, pipelines and terrain data.  [Removed due to blind review].

The applicability of VR techniques for 3D geometric CAD models has been restricted to design review, virtual prototyping and marketing purposes, mainly in the automotive and aircraft industries. More recently, 3D CAD models are starting to show their potential in VR applications for diverse purposes, such as ergonomic studies, safety training for Health, Safety Environments (HSE), and visualization of physical simulations, project documentation and real-time operational data.

This developed tool is integrated into complete system offering resources for real-time 3D visualization and interaction in CAD models with enough realism and performance to be used for collaborative virtual prototyping, design review, change management systems, training, and visualization of engineering simulations among other activities.

Figure 3. (a) User stand in front of a cave composed by three view screens. (b)Virtual representation of the cave in (a) and user’s position using multi-projection calculations.

The use of stereoscopy for immersive applications in engineering is essential. This is achieved by user's headtracking, multi-projection calculations and the determination of dynamic parameters of stereoscopy [Ware et al. 1998].

Figure 4. Sequence of images showing user’s headtracking and the dynamic generation of viewpoints in the virtual environment.

The headtracking helps to determine the relative position of user to the view screens. This position is the input into the calculations that will build suitable viewpoints. Figure 4 is an example of different views by tracking user's head.

Finally, in our scenario, the proper determination of the parameters of stereoscopy is direct related to the navigation techniques that run in the immersive environment. For example, the operation "Go to" has as a prerequisite to determine the target object to start the navigation. Based on the size of this object, the cave representation is scaled and the stereoscopic parameters (distance of the eyes and the zero parallax distance) are calculated dynamically. These parameters are proportionally scaled based on the cave size. Below is a sequence of figures illustrating the scale of the cave representation based on the target object (Figure 5).

Figure 5. Different cave scales based on a target object. These scales dynamically change the stereoscopy parameters (eyes distance and parallax zero distance).

5. Application Scenarios

## 5.1 Collaborative Risers Analysis Workflow

An important step in the deep water oil exploitation is the elevation of the oil from depths over one thousand meters up to the surface. This elevation process is supported by offshore floating platforms. The platform design is complex and extremely important due to their building costs and inherent risks.

Oil platforms use ascending pipes, called risers, tubular structures that convey fluids (oil and/or gas) from the wellhead on the sea floor to the platform's separator system tanks [Senra et al 2002A 2002B]. The risers are connected to the platform using special connections called “joints”. To certificate the operation of the risers for their entire lifecycle (20, 30 years or so), simulations of the stress applied to the riser system are conducted based on meteo-oceanographic data about wind, tide and water currents. In order to avoid operational problems, simulations are made under extreme environment conditions to test against stress resistance. In particular it is important to perform fatigue analysis studies to evaluate the most critical regions of the risers affected by cyclical stress in order to guarantee its integrity during its lifetime.

As a result of their complex behavior, the development and application of rigorous structural and fatigue analysis tools for offshore platforms and risers installed by Petrobras, the Brazilian government-owned oil company, has become a relevant task to engineers.

The analysis of risers requires the usage of a suite of standalone programs. Each one of these programs is executed to perform a specific activity during the entire analysis process. Since the analysis process is complex and handles a large amount of data files, this simulation is very hard to be executed and managed manually. Furthermore, as the input file for one program is the output of another one, it is necessary to make countless data manipulations and transformations using extra tools or program new scripts to accomplish these tasks. Again, this is not only expensive in terms of time, but it can also be error prone.

In this context the use of scientific workflow to to better represent and execute that sequence of simulations is an important feature of the system.

Figure 6: Riser Analysis workflow on BPEL Designer.

We have defined an Anflex-based BPEL Scientific Workflow for riser analysis controlled by the BPEL engine for automating the validation process and certification of riser analysis. The workflow integrates the execution of the following services: Ocean Service, Anflex Service e Grid Job Service. In Figure 6 we show the final version of the Riser analysis workflow in a BPEL designer.

The workflow starts with an Anflex base-case, where the basic configuration of the experiment is defined such as a production unit, riser’s geometry, soil bathymetry, etc. Anflex Service receives user input parameters from BPEL designer and is responsible for creating different loading cases according to the different meteo-oceanographic conditions provided by the OceanService. After that, BPEL instructs CEE GridJob Service to communicate with Condor to submit jobs for executing the Anflex simulation program on the available nodes of the Numerical Grid.

Figure 7: Riser Analysis (CAD + VideoConference).

3D Annotations can also be created by the users. In Figure 7 two 3D annotations were created automatically, showing the extreme points (maximum and minimum values) of a selected force or strength in the riser. The third 3D annotation was created by one of the users to register some important observation made in this collaborative session.

Figure 8: Closer look on an element of the riser

Among other resources, it is possible to playback the simulation, examine pipes, sea waves and ship movements, and track elements in the risers that are subjected to extreme conditions (e.g., high stress values). It is also possible to select any element in a riser and examine it carefully; especially those elements in places subjected to great stress, such as the joints connection and the TDP (Touch Down Point). In Figure 8, the users are looking closely to the behavior of a selected element in the riser (green ring), they can also follow the movements of the element playing the simulation on the timeline bar.

At the end of the session both users will have all the information attached to the model. This information represents the state of the collaborative visualization session and can be persisted in a file that can be loaded again in the future to reconstitute the scenario that was analyzed.

Figure 9: Users in a collaborative visualization session.

Figure 9 shows another collaborative visualization session now with another set of risers. The picture shows the white 3D annotations created automatically, and two other annotations created by each user, making comments about different elements in those two risers.

Observe that in the users tab, we have the awareness mechanism showing information about the status of the user (online, offline) and its role in the session (coordinator or participant).

Figure 10: Users monitoring the behavior of marine buoyant

In Figure 10, we show an engineering project where the users want to study the movement of a buoyant, the usage of the buoyant is to reduce the stress that are submitted the risers. Through the use of the buoyant we can decouple the movement of the platform hull and the movement of the riser system monitoring the distance between the buoyant and the platform and also are observing the behavior of some force on the risers.

## 5.2 Design Review Workflow

Design review is the process of checking the correctness and consistency of an engineering project while making the necessary corrections to it. CEE is very helpful in this process allowing evaluating the safeness of different emergency escape pathways in case of an emergency.

The Design Review workflow is a simplified version of the riser analysis workflow, where the workflow engine invokes Service Coordinator to create a Collaborative Visualization Session with the support of Videoconference Session according to the user’s choice. In this session the users manipulate engineering artifacts and create 3D annotations and make 3D measurements in the model.

Figure 11:  Maintenance plan enriched with annotations.

Object manipulation is an important resource in design review. The ability of moving, rotating and scaling objects is important for various purposes such as joining different models in a scene, viewing hidden portions of the model, planning the placement of a new piece of equipment on a plant, and simulating a maintenance or intervention operation in a process plant are also valuable tools. Moreover, integration with a CAD database is useful to allow users to create annotations on the model emphasizing critical parts (Figure 11). It is also possible to show comments attached to objects, which can be used as recommendations for project management.

Figure 12:  Measurements in 3D in a CAD model

Figure 12 shows some measurements taken for planning the movement of a large tank on production unit. Users create 3D annotations to guide the maintenance procedure  and can also create an animated path showing the entire operation.

## 5.3 Virtual Guided Tour

In the Virtual Guided Tour, a user follows the movements of another user, sharing the same view of the model. Figure 9 shows another collaborative visualization session. The pictures show both users following a 3D path passing through important points and at the end an annotation is created to mark some important event on the platform, maintenance or commission of new equipment could be programmed.

During VR Collaboration three types of sessions may exist: Informal, Classroom and Lecture. So for each type of session the user has a status that is determined according to its role (coordinator or participant). In a Virtual Guided Tour the coordinator is in a state that he sends camera movements and must ignore any camera movements from the other users (SendOnly), while all the other participants are in a state that they can only receive commands and cannot send any camera movements (ReceiveOnly).

The awareness mechanism shows the icons of each user with 2 green lights, one for input and another for output. When changing to a Classroom or Lecture collaborative session the state of the coordinator and participant changes accordingly, as shown in the Figure 13 and 14. Observe that the awareness mechanism changes the icons of each user accordingly, the coordinator change its state to SendOnly (upper arrow green, lower arrow red) while the participant change its state to ReceiveOnly (upper arrow red, lower arrow green).

It is also possible to request the coordinator role, but only in a Classroom collaborative session, in a Lecture session this possibility is forbidden.

Figure 13:  Virtual Guided Tour

Figure 14: User on a Cave system during VR Collaboration doing a virtual guided tour.

6. Conclusions

A collaborative environment was developed to optimize the execution of Large Engineering Projects such as Offshore Engineering projects. Upon the integration of VR technologies into the workflow of the team workers we expect to improve the use of VR in Offshore Engineering projects. It is clear that visualization resources improve the quality of engineering projects, but users do not want to spend their time preparing the content to be visualized in other system, like an immersive multi-projection environment. simplifying the daily job of the engineers, from running simulations on a Grid through visualizing its results on an immersive environment or on a desktop through the Portal just accessing a web page and sending commands to the server.

From the VR and Visualization point of view, the solution approach treats them as first class tools, exploring their potential for facilitating information exchange and common understanding of complex problems.

The perspectives for the future is that many other organizations are going to start to use Scientific Workflows and this will become a common solution in high complex enterprises that have several areas that must be integrated and synchronized.

Acknowledgement

Tecgraf is a laboratory mainly supported by Petrobras. Alberto Raposo receives a grant from FAPERJ E-26/102273/2009.

References

Cruz-Neira, C., Sandin, D. J., DeFanti, T.A., Kenyon, R.V., AND Hart, J.C. 1992. The Cave: audio visual experience automatic virtual environment. Communications of ACM, 35-6.

Callahan S. P., Freire J., Santos E., Scheidegger, C. E, Silva, C. T., AND Vo, H. T. 2006. VisTrails: Visualization meets Data Management. In Proc. ACM SIGMOD International Conference on Management of Data, 745-747.

Deelman, E., Gannon, D., Shields, M., Taylor, I., (2009), Workflows and e-Science: An overview of workflow system features and capabilities, Future GenerationComputer Systems, v. 25, n. 5, p. 528-540.

Dodd, E. J. 2004. Visualization and Collaboration for the On Demand Upstream Petroleum Enterprise. IBM, Toronto, CA.

Eclipse BPEL Project. www.eclipse.org/bpel/

Ellis, C.A. 1999. Workflow technology. In Computer Supported Co-operative Work, M. Beaudouin-Lafon Ed., vol. 7 of Trends in Soft Series, John Wiley&Sons, pp29-54, Chichester,.

Ellis, C.A., S. J. Gibbs, G. L. Reins, 1991. Groupware: Some Issues and Experiences. In Communications of the ACM, vol 34, no. 1, January.

Gruchalla, K. 2004. Immersive well-path editing: investigating the added value of immersion. In Proc. IEEE VR, VR04, 157–164, Chicago, USA.

Houstis, E. N. Gallopoulos, E., Bramley, R., and Rice, J. R. 1997. Problem-Solving Environments for Computational Science. In IEEE Comput. Science and Engineerin, 4(3),18-21.

Klie, H. Bangerth, W., Wheler, M., Parashar, M., AND Matossian, V. 2004. Parallel well location optimization using stochastic algorithms on the Grid computational framework. In Proc. 9th European Conference on the Mathematics of Oil Recovery (ECMOR IX).

Kreylos, O., Bernardin, T. ET AL. 2006. Enabling Scientific Workflows in Virtual Reality. In Proc. ACM SIGGRAPH Virtual-Reality Continuum and its Applications in Industry, VRCIA, 155-162.

Lee, W., McGough, S., Newhouse, S., Darlington, J. 2004. A standard based approach to job submission through web services. In Proc. of the UK e-Science All Hands Meeting, pp. 901–905. UK EPSRC.

[BLIND]

Mourelle, E. C. Gonzalez, E. C., and Jacob, B. P., 1995. ANFLEX - Computational System for Flexible and Rigid Riser Analysis. In Proceedings of the 9th International Symposium on Offshore Engineering.

S.F. Senra, F.N. Correa, B.P. Jacob, M.M. Mourelle, and I.Q. Masetti, 2002a. Towards the integration of analysis and design of mooring systems and risers, part I: Studies on a semisubmersible platform, In OMAE 2002, v. 28046

F.N. Correa, S.F. Senra, B.P. Jacob, I.Q. Masetti, and M.M. Mourelle, 2002b. Towards the integration of analysis and design of mooring systems and risers, part II: Studies on a Dicas system, In OMAE 2002, v. 28151

OASIS Web Services Business Process Execution Language (WSBPEL) www.oasis-open.org/committees/tc_home.php? wg_abbrev=wsbpel

Paraview website. 2011. http://www.paraview.org

Parker, S. G., MILLER, M. HANSEN, C. D., AND JOHNSON, C. R. 1998. An integrated problem solving environment: The SCIRun computational steering system. In Proc. 31st Hawaii Int. Conf. System Sciences , HICSS-31, 147-156.

Paventhan, A. , Takeda, K., Cox, S.J., AND Nicole, D. A. 2006. Leveraging Windows Workflow Foundation for Scientific Workflows in Wind Tunnel Applications. In Proceedings of the 22nd International Conference on Data Engineering Workshops, ICDEW'06, Atlanta, Georgia, April.

[BLIND]

[BLIND]

Simon, A. and Scholz, S., 2005. Multi-Viewpoint Images for Multi-User Interaction. In Proc IEEE Virtual Reality Conference, VR '05, pp. 107-113, March,.

Simon, A., 2005. First-person experience and usability of co-located interaction in a projection-based virtual environment. In Proceedings of the ACM Symposium on Virtual Reality Software and Technology, VRST’05, Monterey, November 7-9, 23–30, CA, USA.

Tufte, E. 1983. The Visual Display of Quantitative Information. In Graphics Press, Cheshire, Connecticut.

VRGeo Consortium Project. 2011. http://www.vrgeo.org

Weske, M., Vossen, M., Medeiros, G., Pires, C. B. 1998. Workflow Management in Geoprocessing Apps. Tech. Report No.04-98-I, Univ.Muenster, Germany.

Ware, C., Gobrecht, C., And Paton, M. 1998. Dynamic adjustment of stereo display parameters. In IEEE Transactions on Systems, Man and Cybernetics, vol. 28, pp. 56–65.