# A Virtual Touch Interaction Device for Immersive Applications

___________________________________________________________________________

Abstract

Traditional interaction devices such as mouse and keyboard do not adapt very well to immersive environments, since they were not ergonomically designed for it. The user may be standing or in movement and these devices were projected to work on desks. Moreover, in the current interaction model for immersive environments, which is based on wands and 3D mice, a change of context is necessary every time to execute a non-immersive task. These constant context changes from immersive to 2D desktops introduce a rupture in the user interaction with the application. The objective of this work is to study how to adapt interaction techniques from touch surface based systems to 3D virtual environments to reduce this physical rupture from the fully immersive mode to the desktop paradigm. In order to do this a wireless glove (v-Glove) that maps a touch interface in a virtual reality immersive environment was developed, enabling it to interact in 3D applications. It has two main functionalities: tracking the position of the user’s index finger and vibrate the fingertip when it reaches an area mapped in the interaction space to simulate a touch feeling. Quantitative and qualitative analysis were performed with users to evaluate the v-Glove, comparing it with a gyroscopic 3D mouse.

Keywords:

3D user interaction, immersive environments, optical-tracking, wireless glove, tactile feedback.

_________________________________________________________________________________________

# 1. Introduction

In recent years, several research initiatives in the field of virtual reality (VR) have focused on the development of interaction techniques for selection and manipulation of objects, navigation and wayfinding [1][2][3]. Techniques for controlling application parameters were also studied, allowing the change of scalar values and the choice among alternative menu options. However, interfaces for system control in virtual environments have not been studied in more depth and are still an open area of study [4].

Since interface controls are an important part of conventional desktop interfaces, adapting these controls for virtual environments is mandatory. A limited number of control widgets, like buttons, can be easily accessed, but does not adequately meet the more complex situations such as selection, menu navigation, and alphanumeric input. Thus, all solutions that enable 2D menus adapted to immersive environments face problems as it is necessary to reach a menu item in the interaction space [5].

Some devices were created and enhanced specifically for immersive environments, such as wands, gloves and 3D mice, providing a more appropriate interface to the user of this kind of environment. However, these devices have a high cost, need an often-complex infrastructure (cables, cameras and sensors) and have limitations when faced with situations common in the WIMP (Windows, Icons, Menus and Pointers) paradigm.

Thus, the user is forced to make several changes of context every time it is necessary to accomplish a task which is not supported in immersive mode. These constant changes in the context of immersion introduces a break in the way of user interacts with the application. Furthermore, these changes often force the users to disassemble the immersion apparatus like the HMD (Head Mounted Display) and gloves, forcing them to sit at a table to perform the interaction tasks necessary and having to return to the immersive way. This avoids the use of such application for longer periods of time or demands the support of a second person to execute WIMP related tasks.

When considering the current advances achieved by interfaces based on single or multi-touch gestures, it was realized that might be some potential in the use of such concepts in immersive virtual environments. The literature presents many initiatives of introducing mobile devices as interaction devices in virtual environments [6][7]. This is mainly due the fact that modern mobile devices have these resources integrated and smartphones and tablets are considered quite simple and useful for gesture interactions. However, they still demand the user to hold the device when interacting with the immersive environment.

Based on these ideas, the aim of this paper is to explore the possibilities of using a touch-like interface in an immersive virtual reality environment using a virtual touch screen mapped in the 3D space. Procedures to investigate the advantages and disadvantages of this approach when compared to standard interaction techniques in this class of environments were conducted. A glove was built using electronic components and a 3 DOF (degrees of freedom) IR (infra-red) optical tracking system. The cameras used were those integrated on the Wii Remote control, also known as WiiMote. Actuators were used to create the vibration sensation and radio transmitters for the communication between the glove and the computer. Based on these features it is intended to simulate the feeling of touching in a virtual immersive environment.

The operating principle of the v-Glove is based on tracking the position of the index finger and mapping it to X and Y display coordinates, similar to a traditional desktop mouse. By moving the finger to the right side, for example, the mouse pointer moves on the screen following this same direction. The operation of clicking and dragging objects on the surface works based on the Z reference axis. When bringing the fingertip to a predetermined point in Z, the system detects the intent to click on the object pointed at that moment. To reproduce the movement of dragging, the user just have to keep the fingertip in the touch area while moving his hand in X and Y axis.

A proof of concept was conducted with the tasks of navigation in CAD models, selection of objects and interaction with menus adapted for the software. Qualitative and quantitative analysis were conducted with users divided into groups according to a profile of experience in the use of 3D applications. All tests were video recorded and the interviews were audio recorded for a more detailed analysis.

The remainder of this paper is organized as follows. Chapter 2 describes some related works that serve as reference and inspiration for this research. The proposed solution is presented in details in chapter 3, its software architecture in chapter 4 and interface adjustments in chapter 5. Chapter 6 presents a case study of the proposed solution with the results presented in chapter 7. Chapter 8 presents the conclusions and some future work proposals.

# 2. Related Work

This research was inspired by several projects from different research areas. For this reason, the related work was divided in three sections according to the area considered the most significant contribution of every related work.

## 2.1. Tactile feedback

The system proposed by Scheibe, Moehringer and Froehlich [8] is a tactile feedback device made for the fingertips in an immersive virtual reality application. The system consists of wired thimbles involving the tips of each finger (Figure 1). In order to simulate the contact with a surface, the wire length is shortened to make them to come into direct contact with the fingertips and a vibration is caused in the wire to enhance the user's perception of a tactile stimulus. Both the shortening and relaxation of the wires as the process of vibration is controlled by a micro-controller that receives commands from a virtual reality application. Studies have shown that users preferred the tactile feedback version of the system when compared to another version without feedback, especially when performing direct manipulation tasks in the car interior. Bullion and Gurocak also presented a compact force feedback glove in [9]. Their main objective was to reduce the size and the number of actuators in a haptic glove, increasing user ability to interact with virtual environments.

. Tactile Feedback at the Finger Tips [8].

Our proposal has the advantage of the absence of wires between the glove and the tracking system, which improves user mobility, especially considering the context of highly immersive environments.

Tactile displays can improve interaction in devices as presented in [10]. A series of experiments were conducted base on the development of perceivable tactile icons (called tactons). These tactons are designed to have parameters like: amplitude modulation, frequency, and waveform, creating distinctive effects. This resource improved the interactions with displays of mobile applications. The goal of this research was conveying two-state signals such as ‘on/off’, ‘slower/faster’, or ‘left/right’. The results showed a reduction of the visual burden experienced when users interact with mobile applications.

## 2.2. Finger Tracking Systems

A.R.T. makes tracking systems for immersive virtual reality environments and has a specific device for finger tracking [11] (Figure 2). This glove allows the tracking of user hand orientation and position of up to five fingers at the same time. The communication between the glove and the control center is wireless. A pair of infrared LEDs is used on each finger for tracking. Each of these sets of LEDs emits light at a specific timing, thus allowing the cameras to identify the fingers individually. This solution has no tactile feedback integrated. Grossman, Wigdor and Balakrishnan [12] proposed a similar approach for finger tracking based on IR markers used for interaction with objects in a 3D volumetric display.

. A.R.T. Finger Tracking System with 3 and 5 Fingers [11].

## 2.3. Virtual Touch Screen

Tosas and Li [13] proposed a mixed reality environment that allows users to interact with a virtual touch screen. Techniques of computer vision are used to detect the hand and gestures of typing on a virtual keypad. The idea is that users can interact with elements on a graphical user interface with clicks made with your bare hands on windows, icons and menus similar to windows managers traditionally used in desktop environments. The proposed system consists of an HMD that allows the user to visualize the virtual keyboard, a camera to capture the hand movements and the software to identify and track hand position, which also generates an audible and visual response when keys are pressed. The click detection is done by identifying the shortening of a finger in the image, when the finger was bent (Figure 3).

. Virtual touch screen [13].

Lindeman, Sibert and Hahn [14] proposed 2D interaction based on hand-held windows in virtual immersive environments. They created a testbed called HARP (Haptic Augmented Reality Paddle) whose main idea is to take advantage of the bimanual interaction in a passive-haptic feedback using a physical surface similar to a track pad. The user holds the paddle in his non-dominant hand and touches the surface to interact with the system. 2D UI widgets are drawn on the face of the virtual paddle representing its current physical location. A head tracking system is also part of the proposed solution to adjust the current view according to the user field of vision. The results of the study support the utility of hand-held windows as a general interaction tool, once they move along with the user and can be positioned to allow the user to work effectively. With the added support provided by passive-haptic feedback, hand-held windows can provide an effective interface for manipulating 2D widgets. When compared to these ideas, our work has the advantage of not requiring the user to carry the physical paddle in one hand, once the virtual touch surface can be activated or deactivated when necessary.

Current research on interaction techniques for imersive environments is focused on selection, manipulation, navigation and wayfinding. The literacture presents a few papers regarding system configuration, symbolic input and 3D menu navigation. This paper presents an interaction device designed to perform application control tasks inspired in techniques already used with sucess in other research areas, such as the idea of touch surfaces and haptic feedback. The proposed device was developed over a glove with tactile feedback for interaction in a virtual touch screen mapped in a VR imersive environment.

# 3. Solution

The purpose of this research is to study the advantages of applying similar concepts of the touch and multi-touch interfaces on immersive virtual reality applications. The main focus was the development of a glove to interact with a virtual touch screen in an immersive projection environment. The hardware view of the solution developed is shown in Figure 4. All built elements were designed to be used in environments without the availability of tables for keyboard and mouse, beside other ergonomic features of immersive environments, like the need for user mobility and low luminosity.

. Schema of the v-Glove hardware.

## 3.1. v-Glove (Tactile Feedback)

Different approaches can be considered when designing a glove for interaction, as presented in the survey of glove-based systems in [15]. The v-Glove is made of micro fiber and has been tailored to ensure a proper size for a larger number of users as presented in Figure 6(a). The choice of fabric took into account its thickness and flexibility as it is crucial that the glove does not restrict the movement of the users’ hands or their ability to handle a tracked wand without having to take of the glove. It is important that the glove do not warm the user's hand, even considering being used in temperature control environments. Additionally, the black color helps the contrast of the optical tracking system.

## 3.1.1. Arduino Micro-controllers

The Lilypad Arduino micro-controller is a version of the well-established processor Arduino developed by SparkFun Electronics to be used in clothes [16]. The programmable microprocessor model Arduino Lilypad Main Board 328 is presented in Figure 5(a). The XBee modem is a component supplied by Digi International Inc. and can be coupled to a Lilypad XBee shield for use in wearable circuitry [17]. XBee implements the ZigBee protocol for wireless communication and has a 1mW microchip antenna with a range up to 100 meters and data transmission rate of 250 kbps (Figure 5(b)). The Lilypad Vibe Board component is an actuator capable of vibrate when powered by 5 volts. It is also built into a wearable configuration as shown in Figure 5(c).

(a) (b) (c)

. (a) Lilypad Arduino, (b) XBee and (c) Vibe Board [15].

The XBee module mounted in the v-Gove is configured as a receiver. Another identical module was connected to the control station via USB port, configured as a transmitter. Upon detecting a touch event, the application executed by the control station sends a command to the transmitter XBee, which then forwards it to the receiver XBee attached to the v-Glove.

(a) (b)

. (a) v-Glove Prototype, (b) Modem in the Control Station.

The electronic components responsible for the tactile feedback sub-system were tailored on the glove. The stitching of the components was done using a sewing line with conductive properties specially created for this purpose. The seam line is silver-plated and has electrical resistance of approximately 45 Ohms per meter. Figure 7 shows the schematic circuit stitched on the v-Glove.

. V-Glove Circuit Diagram [18].

A 9V battery provides power for all components of the circuit. The XBee modem receives through its antenna the commands from the XBee modem transmitter. The command is then sent to the transmit port (Tx) which is connected directly to the receive port (Rx) of the Lilypad Arduino microcontroller. The software stored on the micro-controller receives this information and sends signals to pin number 7, which is directly connected to the vibe board. Activating this device causes in the user a feeling of a slight vibration in the fingertip. The duration of vibration may be controlled.

The main principle of the haptic feedback subsystem is to provide a vibration when the user reaches the touch area mapped by the cameras in the physical space. The vibration is active for 300 milliseconds and is interrupted even if the user keeps the finger in the so called click area. Tests were made keeping the vibration while user’s finger is in the click area, but an empirical evaluation considered the vibration intensity uncomfortable for the user when active for longer periods of time. Regarding cursor operation, positioning the fingertip in the click area causes cursor (mouse) pressed event to be generated in the application. The click operation is only completed when the user returns his finger to a position outside the click area, generating a cursor released event in the operational system. This approach is necessary to enable drag based interactions, when there should be a way to tell the system that the movement was started and when it has ended.

## 3.1.2. Application running in the microcontroller

The program that runs in the Arduino is called "sketch", and is written in a programming language similar to C. The structure of a sketch can be divided into three parts: an area for declaration of variables and functions created by the user; the setup() function; and the loop() function.

Setup function is the first to be called when the program starts. It can be used for the initialization of variables and controller ports or for loading third-party libraries. The setup function is executed only once, at startup or when the board is reset. Loop function is called continuously during program execution. The first execution of this function occurs only after the end of the setup function. Below is a simplified sketch with the source code used in the v-Glove:

1	int finger1Pin = 7;

2	int incomingByte = -1;

3

4	void setup()

5	{

6	  Serial.begin(9600);

7	  pinMode(finger1Pin, OUTPUT);

8	  Serial.flush();

9	}

10

11	void loop()

12	{

13	  if (Serial.available() > 0)

14	  {

15	    incomingByte = Serial.read();

16

17	    if (incomingByte == '1')

18	    {

19	      digitalWrite(finger1Pin, HIGH);

20	      delay(300);

21	      digitalWrite(finger1Pin, LOW);

22	    }

23	  }

24	}

Line 1 makes the declaration of variables related to the port of the Arduino microcontroller used (in this case port number 7). Line 2 declares the variable used for reading bytes from the serial port. Lines 6-8 define the mode of operation for the ports used as output.

Line 13 runs the code block if there is information to be read into the serial port, which are read in line 15. Line 17 performs the following blocks of code if the byte read is 1. Finally lines 19-21 write in the output port a signal of high voltage (5V), wait 300 milliseconds and remove the signal from that port.

## 3.1.3. Finger identification

In order to track the finger position an infrared optical tracking solution is used. Among several options evaluated, the 8910 3M reflective tape was selected as an infrared marker. This tape is widely used as an element of urban signs for its high reflective properties for visible light and its high flexibility (for example it can be easily tailored to uniform traffic agents).

Then, at the tip of the index finger of the glove is applied the 3M retro-reflective tape. This tape has the property of reflecting light with greater intensity in the direction of the illuminating source. So it is possible to efficiently reflect IR to the camera if an illuminator is positioned close to the cameras. The IR cameras capture the light reflected by the tape and estimate the position of the user's finger. As the proposed system uses two perpendicular IR cameras, it is important that the retro-reflective tape is visible by both cameras, and then the most appropriate way to position the tape to ensure good visibility is getting them involved in the fingertip. The positioning of the cameras in relation to the user and the tape applied on the glove are shown in Figure 8.

. Set-up of cameras and tape applied on the glove.

## 3.2. Tracking

The camera chosen for the tracking system is the one integrated to the remote control of the Wii video game, known as WiiMote. This camera has a built in filter to capture only the infrared range, with greater sensitivity to wavelengths near 940nm. The Wii Remote uses bluetooth technology for wireless communication. The experiments developed by Chung Lee [19] showed that the resolution and precision of the device is enough for tracking even small points. The specification was unpublished, but it appears that the camera has resolution of 1,024×768@100Hz, and a 45-degree horizontal field of view. The device can track up to four points simultaneously by the integrated hardware, minimizing data transmission over the Bluetooth connection and simplifying development of algorithms.

The main advantages of using the WiiMote instead of a conventional camera are: it is wireless, already has a built in IR filter and have development libraries that provide the X and Y coordinates of the IR points captured. A disadvantage of the WiiMote is that it is able to identify only four points of infrared light simultaneously, which may limit an application that is intended to track all the fingers. This will not be a problem in our case, since the proof of concept proposed in this research only uses the index finger.

As the camera of the WiiMote provides the coordinates of the points in a two-dimensional plane, a second camera was incorporated and positioned laterally to capture the depth axis. Thus two perpendicular cameras compose the final solution.

The first camera is responsible for tracking the axes X and Y. It indicates the position of the user's fingers on these two dimensions. The second camera is responsible for the Z axis and it is positioned at the side of the user and maps the virtual touch surface. When the user's index finger approaches the mapped plan, the software recognizes a touch on the surface and triggers an event of tactile feedback to the v-Glove, vibrating the user's finger.

Other tracking algorithms using epipolar geometry were evaluated [20][21]. They are very useful for tracking complex objects with multiple markers on it, but require several calibration steps. For this research, only a single point is necessary, therefore the orthogonal WiiMote cameras placement provides a more practical solution. One advantage of the perpendicular cameras is that even if the alignment of the cameras is not perfect the system will continue to work.

The WiiMote infrared camera is only sensitive to a certain intensity and wavelength of infrared light. Considering that this application will be used in an immersive environment with low light, an infrared illuminator component is essential. The created prototype consists of eight infrared LEDs of 5mm and a dominant wavelength of 940nm fed by a power supply of 12 volts. The LEDs used are of the same type used for remote control devices, and can be easily found in electronics stores. This component generates infrared light that is reflected by the retro-reflective tape on the v-Glove and then captured by the WiiMote cameras.

An alternative to using an infrared illuminator is the placement of the LEDs directly at the tip of the glove fingers, instead of the retro-reflective tape. In this case the camera of the WiiMote would directly capture the light emitted by the LED. The problem with this approach is that the presence of LEDs on the glove increases the battery consumption, thus reducing its autonomy.

The LEDs used were evaluated to make sure the Infrared emission is not harmful to users, and the values calculated are far below those considered hazardous.

## 3.3. Control Station

The control station of the tracking system and tactile feedback is a regular PC where the developed software executes. This system is responsible for calculating the coordinates X, Y and Z, as well as to activate the tactile feedback subsystem. In addition, the software can interact with the final applications in the immersive environment.

# 4. Software Architecture

The software architecture was divided into five main modules that interact to provide all the features necessary for the v-Glove operation. The application to validate the proposal was developed in Java. Third party libraries were used and they are mostly open source or have consent for use in non-profit research projects.

. Software Architecture Diagram.

The architecture diagram of Figure 9 shows the main modules and the external components which they interact, indicating in each case the protocol used. The VR proxy component interacts with the VR/CAD application through a TCP/IP connection. The Wii Remote connector performs its communication via the Bluetooth protocol, while the Tactile Feedback connector communicates with the glove through the ZigBee protocol (IEEE 802.15.4).

# 5. Menu and Widgets Interfaces

In order to evaluate the usability of the v-Glove, a case study was prepared targeted at a CAD application [22]. A list of graphical user interface components was classified and prioritized according to their use in the CAD application, resulting in the following ones: button, checkbox, slider, combo box and text box. These components were then adapted for its use with the v-Glove. Figure 10 shows the resulting components in the application menus.

. Widgets Developed for Evaluation Tests.

As a way to complement the tactile feedback, a change in the behavior of the mouse pointer was performed. Depending on the distance of the v-Glove over the touch interface, the pointer on the screen changes its colors. Figure 11 shows possible colors of the pointer.

. Pointer colors.

The color of the pointer remains white while the glove is out of range of the camera or when a distance is more than 400 camera pixels from the touch area. The color changes to yellow when the distance is between 200 and 400 pixels, and orange when the distance is less than 200 pixels. The pointer turns red when the user hits the touch area.

# 6. Evaluation

The v-Glove was evaluated using the CAD model presented at Figure 12. This evaluation had three groups of participants with different profiles regarding development and use of 3D applications.

. Virtual Reality / CAD Application.

CAD applications are usually designed for use on the desktop, and are not fully adapted for immersive environments, since their menus and controls are based on the standard WIMP style. All interaction with the software is done through 2D menus and controls (Figure 13), which required an adaptation of context for its use in the interaction paradigm presented here.

. CAD Application Menu Interface (in Portuguese).

The CAD application menus and widgets were then adapted in a more suitable format for interaction with the v-Glove in an immersive environment (Figure 10). This adaptation occurred only in the relevant components for the evaluation performed during this research.

## 6.1. Participants Profile

TABLE I. shows the average age of participants per interest group. Ages varied from 21 to 60 years and, from the eighteen participants, sixteen were male (89%) and two female (11%). All participants were right handed and had the habit of using the mouse with their right hand. This is an important point to be observed, since the v-Glove prototype was designed for right hand only.

Users were divided into three groups of equal size, according to their prior experience in 3D applications: Group E was composed by system analysts who work for at least 6 months in the development of the CAD/VR application used; Group O was composed by systems analysts with some experience in software development for computer graphics; Group N was composed by individuals without experience in developing or using graphical software.

Based of the metrics developed by Nielsen [23], Virzi [24], and Lewis [25] it is proved that 80% of usability findings are discovered after five participants and a much larger number of participants do not really improve the results. That is why the number of 6 people divided in 3 groups with different skils was the best aproache chosen.

Users Age.

|  | Age Average | Younger | Older |
| --- | --- | --- | --- |
| Global | 29 | 21 | 60 |
| Group E | 25 | 22 | 28 |
| Group O | 31 | 26 | 37 |
| Group N | 31 | 21 | 60 |

94% of the participants had indicated having some experience with 3D applications, like: games, 3D modeling tools and CAD viewers. All users in Group O mentioned some knowledge of the CAD/VR tool, but only a superficial contact and not being part of developers or users group.

## 6.2. Resources

The simulation room has two ProjectionDesign evo22sx+ projectors configured in passive stereo through a circular polarizing filter with a 120" anti-depolarizer screen. The computer used has a NVIDIA Quadro FX 4600, an 3.2 GHz Intel Core i7 processor with 12 GB of RAM. Figure 14 shows the room.

Camera

Illuminator

. Evaluation Facility.

## 6.3. Evaluation Procedures

A pilot test was conducted with users with characteristics of "Group N", since it is expected to observe more difficulties in the participants of this group, helping to improve the evaluation process. The main experiment was evaluated by two questionnaires, one before and other made immediately after the simulation. The examiner completes an accompanying sheet during the operation. All participants filled in and signed a consent form that explain the purpose of the tests, their role in the process and provides the option of quitting it at any time if they wished.

The evaluation was divided into three sequences of five tasks to be performed by participants using different devices in each sequence. One sequence is performed with the v-Glove with tactile feedback feature turned on, the other without the use of tactile feedback and the other sequence performed with a gyroscopic 3D mouse [26]. Users had a 2 minutes training phase before using each device.

The five tasks for each sequence are the same, based on the menus in Figure 10. In the first task the user was asked to navigate in a CAD scene and look for a particular object, having to select it. In the second task, the user should select one of the checkbox options and confirm. The third task involved the selection of combo box options. The fourth task was to select the required value in a slider-type control, and in the fifth and final task, the user should correct the value typed into a textbox, entering a new numeric value and selecting the finish button.

After concluding the sequence of tasks, the interaction device is changed and the sequence is repeated. This was done in a random order to avoid annoying the user having to perform exactly the same sequence of tasks every time. In order to reduce the learning effects between tasks, the order of use of the devices did not repeat among users of a same group. Considering that there were three devices and six users in each group, it was possible to determine a different order for each user within the group.

# 7. Results

Tullis and Albert [27] present ten usability study scenarios and the related usability metrics that apply to each type of study. According to their classification, the present research fits into the "comparison of alternative designs" and five metrics are applicable: task success, task time, issue-based metrics, self-reported metrics and combined and comparative metrics.

## 7.1. Task Success

The first analysis has been done on the data captured about the success or failure in performing the tasks required. Most users have no difficulty in completing the tasks. 100% of users completed the tasks performed with the 3D mouse while using the glove with tactile feedback the success rate of task 2 was 89%, and when the glove without tactile feedback was used the success for tasks 2 and 3 were 94%.

## 7.2. Task Time

The average task for each group is presented in Figure 15. The glove without tactile feedback has the worst average performance, followed by the glove with tactile feedback and finally the 3D mouse. Moreover, the time to accomplish tasks decreases as the level of experience in 3D of the users.

. Task Time Average per Group.

## 7.3. Issue-based Metrics

During the experiment, the evaluators collected information on situations that occurred during the execution of tasks. The issues were classified by frequency, priority (high, medium or low), category and rating (frequency + priority). Figure 16 shows the four usability issues ranked with the highest rating among all observed during the experiment.

. Usability issues per frequency and priority.

The main problems were:

- Lack of precision in Z axis – when moving the hand to reach the virtual touch surface it is difficult for the user to prevent variations in the X and Y axis. For this reason, when the pointer reaches the click area its position might be different from the original one, causing the click to be performed in a different location than the expected;

- Lack of precision on the slider - users reported difficulties to place the slider indicator in the correct position. This happens because, depending on the used scale, the distance between two markers on the rule was small enough so minimal variations in finger position prevent its placement in the desired location;

- Fatigue in the arm and hand - the suggested position to use the v-Glove requires the user to keep its arm raised, causing some users to complain about fatigue in the arm and hand;

- Difficulties to distinguish cursor colors - users reported difficulties to notice the difference pointer colors, particularly between orange and red, resulting in perception issues in the visual feedback, especially when using the v-Glove without tactile feedback.

The results of the usability issues metric indicate that 58% of issues observed during the tests are related to problems in the operation of the interaction devices. Regarding the percentage of the usability issues, only 7% of them are unique to the mouse, while 72% happens in either versions of the glove. The remaining 21% refer to issues observed in both devices, like posture problems or fatigue situations.

## 7.4. Self-reported Metrics

From the data collected in interviews, it was possible to examine self-reported metrics quantitatively and qualitatively. Completion of the questionnaires with questions on the Likert scale [27] is the basis of quantitative analysis presented below.

## 7.4.1. Quantitative Analysis

Figure 17 shows the mean score for each device usability, where the 1 means "very bad" and 7 "very good". The results of the analysis were consistent with results observed in the task duration metrics. The v-Glove without tactile feedback was the worst, followed by the glove with tactile feedback and the 3D mouse.

. Mean Points per Group.

## 7.4.2. Qualitative analysis

Since most of the users have some experience in working with graphical software, during the interviews contributions from the participants were captured. The most common suggestions were:

- Increase the scale of the movement relative to the pointer on the screen;

- Use the click of the first contact with the touch area;

- Detect the click from a minimum variation in the Z axis after reaching the touch area;

- Decrease the intensity of vibration;

- Change the way the click is made to reduce hand and arm fatigue;

- Click through a pinch gesture;

- Recognize click movement from a relative location since the original finger location and not absolute as in the current version;

- Convert the slider widget to a vertical orientation;

- Provide some haptic response then the cursor reaches the area of a widget that supports interaction;

- Provide progressive tactile feedback;

- Include a click button in the v-Glove;

- Keep vibration while the finger is in the click area.

A general perception from the users was about tasks that require some precision, like adjusting the slider widget or selecting an object in a small area of the screen. These issues are partially related to characteristcs of the proposed device, especially because the v-Glove was designed for free hand movements in space, where it is hard for a person to have movements with precision for a long time. However, improvements in the proposed widgets can be made to improve its usability. The slider widget could be redesigned in a vertical axis, increasing the corresponding interaction area for the v-Glove.

## 7.5.  Discussion

The task time metrics provided information with statistical relevance confirming that there are differences in task duration average for different devices. Despite a slight tendency towards a better performance according to the level of 3D experience of the users, the analysis of variance (ANOVA) of two factors with replication determined that this difference has no statistical significance when comparing the performance of one group over the other (F(3.20) = 2.56, p < 0.09). The same technique also confirmed that the time differences between the devices are indeed significant (F(3.20) = 19.19, p < 0.01), confirming the results obtained in earlier observations, when the mouse was expected to have the best performance followed by the v-Glove with tactile feedback.

Based on the results of the usability issues metrics it is also possible to foresee that some effort focused in reducing the usability issues related to the v-Glove might result in improvements in the glove indicators compared to the mouse.

The tactile feedback had also a considerable impact in the v-Glove. The idea behind the inclusion of two versions of the v-Glove in the tests was to study the influence of the tactile feedback in the glove usability. The results of the ANOVA (two factors with replication) applied to task time performance indicate a relevant statistical difference in favor of the v-Glove with tactile feedback (F(4.17) = 5.23, p < 0.03) when compared to the v-Glove without feedback.

# 8. Conclusions and Future Work

In this work, an interaction device for immersive virtual reality environments, called v-Glove, was proposed and developed. The v-Glove is a device that allows a user to interact with virtual reality applications in a natural way through the movement of the hand in 3D space. The selection and manipulation of objects is done by bringing the hand to a touch area mapped in the virtual space. As proof of concept, an application capable of interacting with CAD data was adapted for an immersive environment. Some usability studies of the glove with quantitative and qualitative assessments were conducted with three groups of users with different profiles of 3D knowledge.

The use of gloves as an element of control pointer on the screen is more intuitive than the mouse itself, but because the interaction tasks supported by windowing applications have already been mapped to a 2D model to facilitate its use with the mouse, the graphical user interface has to be adapted. Applications designed for the standard WIMP can hardly be used in such an environment without some kind of adaptation.

Regarding the graphical user interface components studied, it was found that the slider widget is the hardest to be adapted, regardless the device used for interaction. In the combo box component some interaction problems were also identified, since the user requires at least two clicks to select an option. Alternative formats as those proposed by Gerber and Bechmann [28] and Dachselt and Ebert [29] might be considered. These authors suggest the adoption of respectively a ring menu and a cylindrical menu (called collapsible cylindrical trees).

During the evaluation of the tactile feedback it was noted that this is indeed essential to the v-Glove operation, a result confirmed in comparative testing of the glove with and without this feature. Some improvements can still be made to this functionality, such as support to the varying intensity of the vibration and the progressive implementation of tactile feedback to better fit the user expectations.

One of the most important features to be included is the support to multi-touch interaction and feedback. Recognition of gestures for the interaction tasks is another important feature to be considered. A more complete second glove is being produced to be used for left hand users and also allowing interaction with both hands simultaneously. This will improve the interactions, especially for tasks such as manipulating 3D objects in the scene. Once the 3D mouse used in our evaluation is a commercial product, building the v-Glove over a commercial haptic glove as the one provided by [30] was also considered, as the use of Kinect [31] instead of the WiiMote as tracking device that should be considered in future works.

Another resource planned is the clicking through the movement of pinch between the thumb and fingers, as found in the pinch gloves [32]. The glove would have to be modified including two electrical contacts at the thumb and index fingertips to close a circuit when in contact with each other.

The system is also being ported to a highly immersive system [33]. Due the larger dimensions, new studies are being conducted in order to better find a position for the WiiMotes, not occluding the user view (Figure 18).

. High Immersive System Being Tested.

# Acknowledgments

Tecgraf is a laboratory mainly funded by Petrobras. Alberto Raposo receives a grant from FAPERJ E-26/102273/2009.

# References

F. Bacim, D. Bowman, M. Pinho, Wayfinding Techniques for MultiScale Virtual Environments, IEEE Symposium on 3D User Interfaces, pp 67-74, Louisiana, EUA, 2009.

A. Kulik, J. Hochstrate, A. Kunert, B. Froelich, The Influence of Input Device Characteristics on Spatial Perception in Desktop-Based 3D Applications. IEEE Symposium on 3D User Interfaces, pp 59-66, Louisiana, USA, 2009.

S. Smith, S. Du’mont, Measuring the Effect of Gaming Experience on Virtual Environment Navigation Tasks. IEEE Symposium on 3D User Interfaces, pp 3-10, Louisiana, EUA, 2009.

D. Bowman, C. Wingrave, Design and Evaluation of Menu Systems for Immersive Virtual Environments, IEEE Virtual Reality Conference, pp 149-156, Yokohama, Japan, 2001.

C. Hand, A Survey of 3D Interaction Techniques. Computer Graphics Forum, Volume 16, Issue 5, pp 269-281, December 1997.

Jian Chen, Michael A. Narayan, Manuel A. Perez-Quinones, The Use of Hand-held Devices for Search Tasks in Virtual Environments, Proceedings of 3DUI, IEEE Virtual Reality, pp 91-98, 2005.

P. G. de Barros, R. J. Rolleston, R. W. Lindeman. Evaluation of Multi-Touch Techniques for Physically Simulated Virtual Object Manipulations in 3D Space, Proc. of the 6th IASTED Int'l Conf. on Human-Computer Interaction (HCI2011), 2011.

R. Scheibe, M. Moehring, B. Froehlich, Tactile Feedback at the Finger Tips for Improved Direct Interaction in Immersive Environments. Virtual Reality Conference, pp 123-130, Charlotte, NC, March 2007.

C. Bullion, H. Gurocak, Haptic Glove with MR Brakes for Distributed Finger Force Feedback, Presence: Teleoperators and Virtual Environments, Vol.18, No.6, pp 421-433, February 2010.

Huimin Qian, Ravi Kuber, and Andrew Sears, Towards developing perceivable tactile feedback for mobile devices, International Journal of Human-Computer Studies, Volume 69, Issue 11, October 2011, pp 705-719.

ART Fingertracking, http://www.ar-tracking.de/Fingertracking.54+B6Jkw9.0.html. Accessed January 2011

T. Grossman, D. Wigdor, R. Balakrishnan, Multi-Finger Gestural Interaction with 3D Volumetric Displays, UIST, pp 61-70, New Mexico, USA, 2004.

M. Tosas, B. Li, Virtual Touch Screen for Mixed Reality. ECCV 2004 Workshop on HCI, pp 48-59, Prague, Czech Republic, May 2004.

R. W. Lindeman, J. L. Sibert, J. K. Hahn, Hand-Held Windows: Towards Effective 2D Interaction in Immersive Virtual Environments, Proceedings of IEEE Virtual Reality 1999, pp 205-212.

L. Dipietro, A. Sabatini, P. Dario, A Survey of Glove-Based Systems and Their Applications, IEEE Transactions on Systems, Man and Cybernetics, Vol.38, No.4, July 2008.

L. Buechley, M. Eisenberg, J. Catchen, A. Crockett, The LilyPad Arduino: Using Computational Textiles to Investigate Engagement, Aesthetics, and Diversity in Computer Science Education, CHI 2008, pp 423-432, Florence, Italy.

Xbee, http://www.digi.com/products/wireless/point-multipoint/xbee-series1-module.jsp#docs. Accessed January 2011.

Tune Glove Diagram, http://www.flickr.com/photos/jtanenbaum/4156417140/. Accessed January 2011.

Johnny Chung Lee. 2008. Hacking the Nintendo Wii Remote. IEEE Pervasive Computing 7, 3 (July 2008), pp 39-45. DOI=10.1109/MPRV.2008.53.

Lowe, David G., Distinctive Image Features from Scale-Invariant Keypoints, International Journal of Computer Vision, pp 91-110, Springer Netherlands.

P. C. Santos, A. Stork, A. Buaes, C. E. Pereira, Jorge: A real-time low-cost marker-based multiple camera tracking solution for virtual reality applications. J. Real-Time Image Processing 5(2): pp 121-128, 2010.

A. Raposo, I. Santos, L. Soares, G. Wagner, E. Corseuil, M. Gattass, Environ: Integrating VR and CAD in Engineering Projects, IEEE Computer Graphics & Applications, v.29, n.6, pp 91-95, 2009.

J. Nielsen, Why you only need to test with 5 users. Alertbox, 2000, www.useit.com/alertbox/2000319.html.

R. Virzi, Refining the test phase of usability evaluation: How many subjects is enough? Human Factors 34, pp 457-468, 1992.

J.R. Lewis, Sample sizes for usability studies: Additional considerations. Human Factors 36, pp 368-378, 1994.

Air Mouse Go Plus, http://www.gyration.com/index.php/us/products/in-air-micekeyboards/go-air-mouse.html. Accessed January 2011.

T. Tullis, B. Albert 2008, Measuring the User Experience, Morgan Kaufmann (ISBN-13 978-0-12-373558-4), USA, 2008.

D. Gerber, D. Bechmann, Design and evaluation of the ring menu in virtual environments. IPT 2004: Eighth immersive projection technology workshop, Ames, IA, USA, 2004.

R. Dachselt, J. Ebert, Collapsible cylindrical trees: a fast hierarchical navigation technique. Proceedings of the IEEE Symposium on Information Visualization 2001 (INFOVIS’01). Washington, DC, EUA: IEEE computer society; 2001. pp 79–86.

CyberGlove, http://www.vrealities.com/cybertouch.html. Accessed April 2011.

Kinect, http://www.xbox.com/en-US/kinect. Accessed September 2011.

D. A. Bowman, C. A. Wingrave, J. M. Campbell, and V. Q. Ly, Using Pinch Gloves for both Natural and Abstract Interaction Techniques in Virtual Environments, Proceedings of HCI 2001, pp 629-633.

C. Cruz-Neira, D. Sandin, T. Defanti, R. Kenyon, J. Hart, "The CAVE: Audio Visual Experience Automatic Virtual Environment," Communications of the ACM, Vol. 35, No. 6, pp 64-72, June 1992.